Z32.DOC   by   Neil R. Koozer, Kellogg Star Rt. Box 125, Oakland, OR 97462

phone (503)-459-3709

Z32.COM  is  a cross assembler that runs on Z80 CPM and assembles NSC  Series

32000 source code.

The files in this package are:

Z32.COM      The cross assembler

Z32.AZM      Z80 source code for Z32.COM

Z32.DOC      Documentation for Z32.COM

Z1.COM       Z80 assembler to assemble Z32.AZM

Z1.AZM       Z80 source code for Z1.COM

Z1.DOC       Documentation for Z1.COM

MATH.A32     Module containing floating point transcendentals and some I/O

COMPLEX.A32  Module containing complex arithmetic and complex transcendentals

NUMBER.COM   Z80 utility to display IEEE format of floating point numbers

RUNTIME.OVY  Mix-C runtime support for NUMBER.COM

NUMBER.C     C source code for NUMBER.COM

Example 32000 programs to further document usage of the above programs.

DIS32.A32    A 32000 disassembler

TS.A32       Interactive progm illustrating usage of MATH.A32 functions

TC.A32       Interactive progm illustrating usage of COMPLEX functions

SAVAGE.A32   Savage benchmark

APOLLO.A32   Interactive space flight to moon & back (unfinished)

APOLTEXT.A32 An include file for APOLLO.A32

The  above  listed  items  are  hereby released  to  the  public  domain  for

unrestricted  usage.   The only thing you can not do is to copyright  any  of

those items as your own or to otherwise restrict their free usage.

When distributing this code, please include all the above listed files.

Anyone desiring a new copy from me,  please send $8 for a CPM 8" SSSD disk or

$7 for an IBM 360k format disk.  The IBM format is for mailing only; the code

will not run on the IBM.

Anyone  who  sends  me  a  useful  suggestion or  bug  report  for  Z32  will

automatically receive a complementary copy of the next update.  (unless I get

deluged)

................................OVERVIEW.....................................

This  assembler  breaks with tradition in a number of areas.   The reason  is

that  I  sympathized  with my poor little  microprocessor,  which  works  its

fingers  to  the bone while I spend huge amounts of time waiting  for  it  to

finish.   Tradition  says  "Why should I do this when I have a computer  that

should  be doing it".   Assembling or compiling a program is an arduous  task

for a microprocessor, and I feel a need to give the poor little thing a hand.

I have, therefore, lightened its load by:

1)  eliminating traditional features that I never use

2)  using efficient algorythms

3)  tayloring some syntactical details to minimize processing time

OUTPUT

I  think  that hex-format intermediate files were invented to cope  with  the

fact  that paper tape was not 8 bits wide.   Since we now have 8-bit  storage

media,  I  see no need for these cumbersom files.   This assembler outputs  a

conventional  .PRN  listing file and an executable .E32  file.   Both  output

files  are  optional,  and  there is no provision for the generation  of  any

intermediate files such as Intel hex.   In the executable file, the gaps left

by ORG and DS statements are filled with 00's except at the beginning of  the

file.   If you put a bunch of buffers at the beginning,  they will not add to

the stored code in the E32 file.   An up-loader such as that in the beginning

of Z32.AZM or APOLLO.A32 could move the code to its proper location.

ONE PASS

I  think that two-pass assemblers were invented to cope with  the  sequential

nature of paper tape output.  Since we now have random access media, I see no

reason  for  two passes.   This assembler is always one-pass.   There  is  no

provision  for two-pass operation.   A label can be forward  referenced  only

once  before  it is resolved,  so it is best to arrange the program  so  that

subroutines,  jump destinations, and data objects are defined before they are

used.   In  the  few  places  where this can not  be  done,  we  use  forward

referencing.   For  the very small forward jumps,  we use psuedo-labels  (see

below).

NO MACROS

Since I never use macros, I don't want the processor to spend time looking in

the  macro table before doing the mnemonic search.   Because of the speed  of

this assembler,  I expect to be able to conveniently assemble large  programs

that  fill  all  free memory with the symbol table.   Eliminating  the  macro

feature frees up some valuable memory.   If you use macros for text entry,  I

suggest  the use of a translator to translate macro-infested source  listings

to  real  source listings.   If you use macros to  repair  the  assembler,  I

suggest   correcting  the  assembler's  source  code  and  re-assembling  it.

Hopefully,  Z32 will never be distributed without its Z80 source code and the

Z80 assembler that was used to develop it.

NO LINKING

The  streamlining  of  this  assembler  allows it  to  run  faster  than  the

traditional  linkers.   Therefore,  linking is performed at the  source  code

level  with  the INCLUDE statement.  Some benefits of this method are:

1)  easier alteration of modules

2)  correct addresses in listing files for easier debugging

3)  no need to keep track of OBJ files that would also clutter disks

4)  all addressing modes and all assembly-time arithmetic available

without regard to module boundaries

NUMBERS

Literal  numbers must be either decimal or hexidecimal.   I am dumping  octal

and  binary  literal formats because I have never used them in the  10  years

that I have been doing assembly programming.   A word that starts with 0 is a

hexidecimal  integer.   A  trailing 'H' is not recognized.   I  always  hated

making the decision between look-ahead and back-tracking,  so I'm eliminating

the  suffix.   Floating  point  literal numbers are expected  to  be  already

converted  to IEEE binary format.   They are represented by a period followed

by  hexidecimal  characters  representing  the  binary  value.   The  utility

NUMBER.COM is used to display the IEEE format of any arbitrary number.

SPEED

When the listing output is suppressed, Z32 assembles DIS32.A32 (my simple 32k

disassembler) at about 28700 lines per minute.   This is 51 (fifty-one) times

as  fast  as the commercial cross assembler that I previously  purchased  and

discarded.  This doesn't even count the linking time that is required for the

commercial  assembler.   The  equivalent clock speed of my Z80 is about  2.57

Mhz, so a good 6 Mhz Z80 board would do over 1000 lines per second.  When the

listing  output  is not suppressed the speed drops to about 12700  lines  per

minute or about 31 times as fast as the commercial assembler.

.............................INVOKING ASSEMBLER..............................

Typing  'Z32 NAME.XXX' will cause the assembly of the source  file  NAME.A32.

What looks like an extention in the command line is a list of drive names.

The  first  character after the period designates the drive on which to  find

the  source  file.   An invalid drive name is used to  indicate  the  default

drive.

The  second character after the period designates the drive on which to store

the  executable output file.   An invalid drive name is used to indicate  the

default drive.  A 'Z' is used to prevent the creation of the executable file.

The executable output file uses the extension E32.

The  third character after the period designates the drive on which to  store

the listing file.   A letter greater than the highest-lettered valid drive is

used  to prevent the creation of the listing file.   If this (high) letter is

other  than 'Z' the listing will be displayed on the  console.   A  character

lower  than  the lowest-lettered valid drive is used to indicate the  default

drive.

Examples:

command          source drive   output drive   listing device

------------------------------------------------------------------

Z32 NAME            default        default        default

Z32 NAME.AZX           A             none           CRT

Z32 NAME.ZXZ        default        default         none

Z32 NAME.BA            B              A           default

...................................SYNTAX....................................

This  assembler uses the syntax described in the Series 32000 Instruction Set

Reference  Manual as amplified by the following items:

LABELS

Labels  must begin in the left column and no colon is used.   Label names can

consist  of letters and numerals,  and the first character must be a  letter.

In this context, the characters   ! # $ % _ ?  count as letters, and @ counts

as a numeral.  Upper and lower case are equivalent.  A label name can have an

arbitrary number of characters, all of which are significant; however, a line

of source code cannot exceed about 127 characters.  A label name cannot be R0

thru R7,  F0 thru F7,  or TOS.  The use of EXT, SB, SP, or FP as labels could

result  in some backtracking.    Each label has a 'type' associated with  it,

and there are presently five types:

AAA  = 12345               ;integer constant

GGG  =F .BF23891E          ;float constant

HHH  =L .3FF23344AABBCCDD  ;long float constant

STATIC                ;start of static section

BBB  DD AAA,245,'ABCD'     ;static-base-relative label (or variable)

ENDSEG                ;end of static section

CCC  MOVB R0,R4            ;program-relative label

When  these  different  types  of labels are used  as  operands  in  assembly

statements, the result will be different according to the type of the label:

MOVD AAA,R3    ;the value of AAA will be an immediate operand

DIVL HHH,F4    ;the value of HHH will be an immediate operand

MOVD BBB,R3    ;loads the value at location BBB into R3 using

;disp(SB) mode.

MOVD CCC,R3    ;loads the value at location CCC into R3 using

;*+disp mode (program relative).

BR CCC         ;branches to location CCC.

JUMP CCC       ;jumps to location CCC using *+disp mode.

ADDR CCC,R3    ;moves the address of CCC into R3.

The symbol AAA,  as defined above, is used as a constant in assembly language

statements,  but during assembly-time,  it is a variable in the assembler and

can be altered any number of times by the '=' psuedo-op.

Labels  can be forward referenced in expressions in a  restricted  way.   The

expression  is restricted to effectively adding a forward symbol to the  rest

of  the expression.   The 'E' error code shows up for invalid use of  forward

references  in  an expression.   A label can be forward referenced only  once

before it is resolved.   A second attempt will be flagged with the 'U'  error

code.

PSUEDO-LABELS

Note  that  these  psuedo-labels work differently than those  of  the  Z1.COM

assembler,  so  if  you try to read the source code for Z32,  then  refer  to

Z1.DOC.    I  call  these  things  psuedo-labels  because  they  don't   have

descriptive names and because they don't use the symbol table.   I don't call

them local labels because they have unlimited scope.  They are used primarily

for  the  short jumps where meaningful names are not needed or even  desired.

They are actually embellished versions of the $ symbol.   They consist of two

characters,  the first of which is $.  The second character can be any of the

letters or numerals as described above for labels.  Example:

$0   ADDD R3,R4

ACBB -1,R0,$0

MOVQB 5,R0

$0   BSR DOSUMPN

ACBB -1,R0,$0

Since  these symbols don't go into the symbol table,  there is no  search  to

find  them.   Using  these  symbols prevents a lot of nonsense  symbols  from

bloating  the  symbol  table,  thereby making large  programs  assemble  more

efficiently.    As  with  regular  labels,  a  psuedo-label  can  be  forward

referenced once before it is resolved.  Because these symbols are reusable, a

forward  reference  must be explicitly identified.   This is accomplished  by

replacing the '$' with a '*' as follows:

BCC *A:B       ;*A is a forward reference to $A

MOVB 45,R0

BR *B:B        ;*B is a forward reference to $B

$A   MOVB 73,R0     ;symbol $A resolved here

$B   BSR THERE      ;symbol $B resolved here

....

BR $A          ;they are used normally after being resolved

The assembler issues a 'U' error code if a forward referenced psuedo-label is

referenced  again  before it is resolved.   After it is resolved  it  can  be

referenced any number of times.   If two lines must make forward reference to

the same spot, then two psuedo labels can be used:

BFS *C:B       ;the :B specifies one-byte displacement

CMPB 'S',R0    ;   in forward references only

BEQ *D:B

...

$C

$D   MOVB R0,R5

The  psuedo-labels  can be used in expressions,  but the  forward  references

suffer  the same restrictions as forward-referenced  regular  labels.   Since

these  symbols  do not nest,  there are two separate symbols with  a  nesting

property,  $_ for backward and *_ for forward.   There is no relation between

$_  and *_ as they are two independent symbols.   They are intended primarily

for jump tables:

CASEWN *M:W[R5:W]        ;the 'N' suffix enables the use of

$_   <routine1>               ;   the CASES psuedo-op (see below)

$_   <routine2>

$_   <routine3>

...

$M   CASES $_,$_,$_,...       ;reverse order due to nesting

or

CASEBN $+4[R5:B]

CASES *_,*_,*_,...       ;reverse order due to nesting

*_   <routine1>

*_   <routine2>

*_   <routine3>

...

The  first arrangement will assemble faster because there is only one forward

reference.  Another use is nested loops:

MOVQB 7,R0

$_   MOVW 237,R1

$_   BSR DOSUMPN

ACBW -1,R1,$_      ;this one references the second $_

BSR DOSTUFF

ACBB -1,R0,$_      ;this one references the first $_

The  maximum  nesting depth of $_ and *_ is set by a couple of EQU's  at  the

beginning of the source code of the assembler.   They are presently set at 40

for $_ and 10 for *_.

INSTRUCTION MNEMONICS

These  must  be in upper case.   The assembler does not enforce the use of  a

space or tab after a mnemonic:

MOVDAAA,R0    is interpreted as    MOVD AAA,R0

The  mnemonic state machine simply quits looking as soon as it  unambiguously

finds a valid mnemonic or proves it to be an unknown mnemonic.

INSTRUCTION OPERANDS

In  the  assembly  language operands,  any place a  'disp'  is  expected,  an

assembly-time expression can be used.   The same applies to 'short' and other

miscellaneous  numeric operands.   An expression representing a 'disp' can be

appended  with  :B  or  :W  so that if  the  expression  contains  a  forward

reference,  the assembler will be forced to use 1-byte or 2-byte disp format.

If the designated size turns out to be too small,  the assembler issues a 'V'

or 'B' error code on the line where it attempts to resolve the reference.

In general operands, where a 'disp' is expected, the result of the expression

must  have  the 'type' of an integer constant.   The expression  can  contain

relative-address  type  symbols  (including one forward ref) if  there  is  a

subtraction to make the result integer.

A  general operand can also consist entirely of an assembly-time  expression.

In this case,  the result of the expression can be one of several types.   If

the type is that of a constant (int,  float,  long), then the general operand

will be an immediate value.   If the type is SB-relative or PC-relative, then

the general operand will be of the corresponding addressing mode.

In  the external addressing mode,  the second disp can be omitted,  in  which

case the assembler will supply a default 0.

The  operand   EXPRESSION2(EXPRESSION1)  will result in the  disp2(disp1(SB))

addressing mode if the result of EXPRESSION1 is of SB-relative type.

The pre-defined operand '*' is a synonym of '$' and represents the address of

the  beginning of the current instruction.   It can be used in  assembly-time

expressions.

In  the relative branching instructions such as Bcond and  ACBi,  the  'disp'

operand can be of integer constant type or of PC-relative type.

BR 4          ;branch 4 bytes beyond the start of this instruction.

BR $ + 4      ;same result.

BR HHH + 4    ;branch 4 bytes beyond the spot where label HHH was

;created if it was created as a PC-relative label.

BR KKK + 4    ;branch (KKK + 4) bytes beyond the start of this

;instruction if KKK was created as an integer constant.

.................................PSUEDO-OPS..................................

ORG <expr>

Sets  the  program address counter to <expr>.   If the output file is  empty,

nothing will be stored,  otherwise the gap will be filled wtih 0's.   ORG  is

not normally needed because of the position-independence of 32k code.  <expr>

can not contain forward references, and its type can be integer or one of the

address  types.   If the output file is still empty,  then ORG is allowed  to

decrease the program counter as in this example:

Var1 DS 8 ;suppose we have a bunch of objects defined with DS's

Var2 DS 8

...

VarN DS 8

ORG Var1  ;back up to overlay temporary code into variable space

...       ;up-loading code goes here

...       ;once-used initializing code and messages go here

ORG VarN+8 ;make sure the permanent code starts beyond buffers

DS <int expr1>

DS <int expr1>,<int expr2>

Adds  <expr1> to the program address counter.   If the output file is  empty,

nothing will be stored,  otherwise <expr1> number of 0's will be added to the

output file.  <expr1> can't be negative.  The fill character may be specified

by <expr2>.

END

Terminates  assembly  whether or not more source lines exist.   Not  used  in

include files.   END is not needed if the source file has a 01A following the

0D,0A at the end of the last line.

DW   wordlist

Wordlist  is a list of expressions separated by commas.   The result of  each

expression is compiled with the least significant byte at the lower  address.

The  result of each expression must have the type of an integer constant  and

must  be  in  the  range -8000h to FFFFh inclusive.   If an  argument  to  DW

contains a forward reference,  the range is -8000h to 7FFFh inclusive because

it  is assumed that the difference of addresses should be a  signed  integer.

These  range limits pertain only to the assembler's error reporting.   If you

want to do something unusual,  you may ignore the error indicators.   Due  to

the  positive range of DW (with backward references),  it can not be reliably

used  to construct case tables because the CASEi instruction uses  the  table

entries as signed integers and does sign-extending.  (See CASES).

DB   bytelist

Bytelist is a list of expressions or quoted strings separated by commas.  The

result  of each expression must have the type of an integer constant and must

be in the range -80h to FFh inclusive.  If an operand is a string enclosed in

single quotes,  the string is transferred to the output file in sequence.   A

single quote can be included in the string if it is represented by two single

quote marks.  If an argument of DB contains a forward reference, the range is

-80h  to 7Fh inclusive because it is assumed that forward references will  be

dealing with differences of addresses.   DB (with backward references) cannot

be reliably used to build case tables (see DW for reason).

DD <list>

<list>  is  a list of expressions separated by commas.   The result  of  each

expression  is compiled as a 4-byte value with the least significant byte  at

the lowest address.  In DD arguments, an overflow is not flagged as an error.

CASEiN

Nested  CASEi statement.   Same as the CASEi opcode except that the N  suffix

causes  the assembler to store the address and size of the instruction  in  a

small  stack  area for use by the CASES psuedo-op.   Pairs of  CASEiN...CASES

instructions  can  be nested.   The allowed nesting depth is set  by  an  EQU

statement in the source code of the assembler and is presently set at 5.

CASES <list>

Used  for the building of a case table for the most recent  unmatched  CASEiN

instruction.   It  works  like DB,  DW,  and DD except that it  automatically

subtracts  the address of the corresponding CASEi instruction and sizes  each

entry  according  to  the  size of the  CASEi  instruction.   Each  time  the

assembler  sees  a  CASEiN  instruction,  it  stores  the  address  and  size

information  of  the instruction in a location known to the CASES  psuedo-op.

Example:

CASEBN FOOS:W[R4:B]

....

FOOS CASES ROUTINE1,ROUTINE2,ABORT,RETRY,

ROUTINE5,TEST23,PASS,RUN,PUNT

Note that the CASEiN instruction doesn't need a label.   If we get an out-of-

range error using CASEBN, we switch to CASEWN, but we don't have to touch the

case table.  If the list of arguments won't fit on one line, a dangling comma

is  used  to  signify  that the list continues on the  next  line.   In  this

situation  the  word  CASES  can't be repeated on each line  because  of  the

nesting property.

FLOAT <list>

Similar  to  DD  but for floating point values.   Each argument  must  be  an

expression of type float.  Example:

FLOAT .D412A650, THING, .12 AA 23 4E, .D67A 78C4

LONG <list>

Similar  to DD but for long floating point values.   Each argument must be an

expression of type long.  Examples:

LONG .3FF234567B3ED3A5, THIS, .3F F0 00 00 12 A3 24 A0

LONG .3FE0 0000 0000 0000, .BFF56789 BCDEF123

label = <expr>

Sets  the  value  of label to the result of  <expr>.   If  the  label  didn't

previously exist,  it is created.  The label will be of integer constant type

regardless of whether we're in a static segment or a program segment.  <expr>

cannot  contain any forward references.   The type of <expr> can be  anything

but float or long.  In the following fragment

AAA  MOVB R0,R4

BBB  = AAA

the  label AAA is PC-relative or SB-relative type,  while BBB is  an  integer

with a value equal to the address of AAA.

label =F <float expr>

Same as '=' but for floating point type.

label =L <long expr>

Same as '=' but for long floating point type.

IF        (COND is synonym)

ELSE

ENDIF     (ENDC is synonym)

Control  conditional assembly in the traditional manner.   Any  assembly-time

expression  can  be  used as the argument of the IF.   IF statements  can  be

nested.

IFDEF <word>

Same  as IF except that it derives its true/false condition by searching  the

symbol table for the existance of <word> (true = symbol exists).  Example:

IFDEF LineBuff ;see if the line buffer has been already created

ELSE

LineBuff DS 080     ;if not then create one

ENDIF

This allows a module (include file) to automatically use a predefined  buffer

or to create its own if necessary.   This provides a means of putting all the

buffers  in the static section at the beginning of the program so they  won't

be stored in the executable file.

STATIC

Sets  a flag so that in subsequent lines,  labels will be of the  SB-relative

type.

ENDSEG

Resets  the segment flag so that in subsequent lines,  labels will be of  the

PC-relative type.

ALIGN <expr>

Compiles 0's if necessary to even up the program address counter.   If <expr>

= 2, it stops at the first even address; if <expr> = 4, it stops at the first

4-even address; ditto for 8.

INCLUDE <filename>

Causes  the  assembler to assemble the source code from the  file  <filename>

before  continuing  with  the  main file.   If  <filename>  doesn't  have  an

extension,  the assembler will use the default extension .LIB.  Include files

can not be nested.

EJECT

Continue  at  the  top  of the next page.   The title line  will  be  printed

following the form feed.

TITLE <text>

The  text  after the word TITLE will be appended to the end  of  the  default

title which reads "Z32 Ver x.x File xxxx".  The title line will be printed at

the top of each page if the LIST F option is active.

SPAC <expr>

Leave <expr> blank lines in the listing.

LIST ON

LIST OFF

Turn the listing on or off.

LIST <options>

Turn on options (see below).

NLIST <options>

Turn  off options.   Any number of options can be specified on the same  line

with  commas  between  them.   Options  not mentioned in  <options>  are  not

affected.   Inclusion of non-existing single-letter options in <options>  has

no effect.  The available options are listed below.  The ones marked (on) are

on by default.

A      List all bytes produced  in the  assembly of  a  line.   Otherwise

only the bytes that will fit on one line of the PRN file.

F      Format the PRN file with form feeds & a title line on each page.

The number of lines per page is defined by an EQU statement in the

beginning of the source of the assembler.  It is presently 57.

I (on) List lines of conditional code in a false clause.  If off only the

code actually assembled is listed.

O (on) Output assembled code to the binary file.

S (on) Output lines to listing file.

T (on) Output symbol table to listing file.

ON and OFF can be mixed with the single letter options on the same line:

LIST A,ON,T

LIST A,S,T     ;both lines are equivalent.

It is best to keep the ON or OFF at the end of the list because the statement

LIST A,OFF,T

will also turn off T.

..........................ASSEMBLY-TIME EXPRESSIONS..........................

Spaces  and  tabs are allowed between operands and operators in  assebly-time

expresssions.

OPERANDS

In assembly-time expressions, the operands can be:

LITERAL NUMBERS

A  word  beginning  with 1 thru 9 is a  decimal  integer.   A  word

beginning  with 0 is a hexidecimal integer.   Alphabetic characters

in hexidecimal numbers can be upper or lower case.   A trailing 'H'

is  not recognized.   A number beginning with a decimal point is  a

floating  point decimal number.   Octal and binary literal  numbers

are not recognized.

LITERAL STRINGS

A string of up to four characters enclosed in single quotes will be

treated the same as a number in expressions.  In the line

CMPD 'ABCD',0(R4)

the ordering of bytes is such that the 'A' will correspond with the

lowest byte of the other operand in memory.

ANOTHER EXPRESSION

An  expression in parentheses is a valid operand in an  expression.

The nesting depth is limited only by the amount of stack space.  If

this depth is exceeded, the assembler complains and then returns to

the operating system.

LABELS

The  type  of a label determines what happens if it us used  in  an

expression.   Labels of integer constant type can be used with  all

the  operators listed below.   Labels of SB-relative or PC-relative

type  can  be added to an expression that otherwise  contains  only

integer constant operands.   Subtraction of two (similar)  relative

type labels yields a result of constant type.   Any other operation

involving two relative type labels is not allowed.

OPERATORS

The unary operators are all of precedence level 1:

\ .NOT.             ;bitwise invert

.B.                 ;clear the upper 3 bytes to all 0's

.W.                 ;clear the upper 2 bytes to all 0's

.WH.                ;get the high word into the low position

;and clear the upper word.

+                   ;unary + (do nothing)

-                   ;negate the operand

The binary operators are listed with the precedence level at the left

2    **                  ;exponentiation

3    *                   ;multiply

3    / REM               ;similar to QUO and REM in the 32k instruction set

3    DIV MOD             ;similar to DIV and MOD in the 32k instruction set

3    SHR                 ;shift right

3    SHL                 ;shift left

3    SRA                 ;shift right arithmetic

3    RR                  ;rotate right

3    RL                  ;rotate left

4    + -                 ;add & subtract

5    & AND               ;bitwise and

6    ^ OR                ;bitwise or

6    XOR                 ;bitwise exclusive or

7    = > < <= >= <>      ;signed comparison, true = 0FFFFFFFF, false = 0

7    ULT ULE UGT UGE     ;unsigned conparison, true = 0FFFFFFFF, false = 0

.........................FLOATING POINT EXPRESSIONS..........................

Floating  point  expressions can presently contain only one  operand  and  no

operators.   The  type  will be float or long depending on the  routine  that

calls  for  the  expression.   The  operand  can only  be  a  symbol  of  the

corresponding  type  or a literal value with the corresponding  length.   The

format  of  the literal value is the hexadecimal representation of  the  IEEE

binary format with a leading period.   The hexidecimal format of the floating

point  value  can  have  spaces between each pair of  characters  for  easier

reading.  Examples:

.3FF0000000000000                  ;1.00

.40 09 21 FB 54 44 2D 18           ;pi

.3FF9 21FB 5444 2D18               ;pi/2

...............................ERROR REPORTING...............................

When  the  assembler  detects what it believes to be an error in  the  source

code,  it  writes  a  single-letter  error code in the  left  column  of  the

offending  line in the listing file and echos the line to the console.   When

the  error  is  an unknown opcode mnemonic,  the  assembler  compiles  8  NOP

instructions  to  give some opportunity for patching during  debugging.   The

following list describes the meaning of each error code:

B =  Out of range destination for forward references using :B or :W

C =  IF...ENDIF unbalance encountered at end of program.

D =  Duplicate symbol definition.

E =  Expression error.  Usually results from attempting illegal

operations on some of the symbol types.

F =  Format error in building assembly language operands or use of odd

registers for long float values.

I =  Immediate operand used as a destination or as basemode of scaled

indexing mode or as a source for the ADDR opcode.  Or wrong type

of immediate operand.

K =  Attempting to use a pre-defined operand (register name or TOS) as a

label.

L =  Label error.   Usually means trying to assign a value to the  wrong

kind of pre-existing label.

N =  Nesting or de-nesting overrun in the *_ or $_ psuedo labels or

CASEiN...CASES instructions.

O =  Opcode error.  Usually means an unknown mnemonic.

Q =  Miscellaneous errors.  Usually a bad operand.

R =  Resolved forward reference too big to fit in operand. Used in DB,

DW, and CASES.

S =  Syntax error in expression.

T =  Type mismatch or disallowed type.

U =  Unresolved forward reference.

V =  Value out of range for the size of the operand.

Z =  Divide by zero.

IMPLEMENTATION NOTES.   In order to resolve forward references during single-

pass  assembly,  the  object  output  uses demand-paged  virtual  memory  and

multiple  buffers.   In the listing file,  forward references do not show the

resolved value.   To speed the handling of text,  another buffering method is

used for the input,  include, and listing files.  The method essentially adds

some  auxilliary space to the length of the buffer so that you never have  to

worry  about the end of the buffer occuring in the line you are  working  on.

This  obviates many operations and allows some operations to be done with Z80

block move instructions.  Since symbols can be any length, they are stored on

the  heap with no empty space between them.   To facilitate a binary  search,

there is an array of pointers to keep track of where the symbols are located.

When a new symbol is created,  its pointer is inserted into the array in  the

appropriate  (alphabetical)  spot and the rest of the array moved  down.   To

speed the search for mnemonics,  I put them all into a 'state machine'.  This

is faster than a binary search or any hashing method because it never has  to

start  over at the beginning of the next word.   It essentially does a multi-

way branch on each character.   For example,  in the words ADDP,  ADDC, ADDQ,

and  ADDR,  after getting through ADD,  it only uses the fourth character  to

determine  if it has one of these words.   It doesn't have to go through  ADD

again.   The  'state  machine' is also fast because there is  no  looping  or

calling involved; it consists entirely of compares and branches.

INPUT FILE FORMAT

In  the interest of speed,  the assembler places some quality demands on  the

source file.   All lines must be terminated with a 0D,0A,  and all lines must

be  shorter than 128 characters (including the 0D,0A).   The character  after

the 0D can be anything;  the assembler skips it without looking at it.   If a

too-long  line  occurs at the end of the buffer,  the assembler will  try  to

assemble code that lies beyond the end of the buffer.  The file also must not

contain characters with bit 7 set.   The assembler uses the raw characters to

key into jump tables which only have 128 entries.

LOOSE ENDS

Immediate  operands  presently cannot contain forward  references.   This  is

usually not needed since nearly all expressions containing forward references

are supposed to return an address anyway, as in the following line:

ADDR FWDREF+23,R0

The source operand uses PC-ralative addressing mode instead of immediate.

Floating  point  assembly-time  arithmetic and conversion of  floating  point

literal  values will probably wait until I do my native 32k assembler.   That

assembler  will  be  integrated into my high level  language  such  that  any

statement can be assembly syntax or high-level syntax.

I  considered  making  the symbol table a two part affair  with  a  permanent

section  and  a temporary section.   At the end of  each  include  file,  the

temporary  section would be deleted.   At the beginning of each include  file

would  be a list of symbols that should go into the permanent section.   This

scheme  would  allow very large programs to be assembled without  the  symbol

table  growing out of control.   This effect is partially achieved by the use

of psuedo-labels.  As a library module (include file) becomes more mature, it

can safely use more psuedo-labels in place of regular labels.

The  assembler assumes that somebody will load the static base register  with

an  address  corresponding to the starting address of the first statement  in

the source file.   It is presumed that a dumb loader will load the .E32  file

into  some  arbitrary spot in memory and then jump to the first byte  of  the

loaded  code.   The  E32  code will then move itself above  its  static  data

section   and  load  its  static  base  register.    The  program  APOLLO.A32

illustrates  this method.   If an operating system existed,  it would have  a

smarter loader that would avoid the extra move.   In this case, the assembler

would have to supply an offset value in a location known to the loader.

I  haven't been exposed to a regular 32k assembler,  so I don't know what  is

customarily done with that run-time linking stuff.  Is the assembler supposed

to have features to deal with module tables and link tables or do people code

these up with DD's and the like?  Should there be additional symbol types for

things such as frame variables or stack variables?

############################################