Kotekan, an OOPerating system (sorry, but I couldn't resist)

Several years ago, perhaps from boredom but more likely from sheer

lunacy, I designed and wrote an operating system I call Kotekan

(Koh-teh-kahn). The name derives from a Balinese music term for

interlocking melodic lines, something like a musical tapestry. My

objectives for Kotekan were: multi-tasking; flexible, dynamically

alterable device drivers; a simple way to redirect or link devices;

a consistent application program interface for all devices;

recursive, romable, real-time code; and small size. Object-oriented

architecture was not a formal specification, but it turned out to be

a good way to get the job done.

Although I wrote Kotekan in assembly language, it includes essential

object-oriented features: encapsulation, inheritance and

polymorphism. Philosophically, I became an object-oriented

programmer long before object-oriented development software was

readily available. I look upon assemblers and compilers as tools

which inherit important characteristics from other kinds of

tools: They are a means to an end and not an end in themselves, and

they can either help or hinder the successful completion of a task

depending on how they are used. Assemblers don't have all the

features of an object-oriented compiler, but their flexibility

offers the means to complete any programming task. Encapsulation

and polymorphism are easily achieved in non-object-oriented

languages with appropriate data structures. Inheritance and dynamic

(or late) binding require additional routines to handle details that

an object-oriented compiler could process automatically.

Kotekan runs on a home-grown computer based on the undeservedly

obscure 32532 CPU from National Semiconductor, a high performance 32

bit chip with built-in memory management. My system includes 1.5

megabytes of memory, floppy disks, a math co-processor, an IBM AT

compatible keyboard and a Texas Instruments 34010 graphics co-

processor.

Although operating systems perform many functions, this article will

concentrate on managing I/O devices (keyboard, printer, disk drives,

etc.). We will look at how Kotekan's device drivers work internally

as well as the application program interface. For the rest of the

article, "program" will refer to an application program loaded and

run by Kotekan.

Devices as Objects

The duties of an operating system include initializing devices,

error handling, moving characters between the device and the program

(reading a character from the keyboard, or displaying a line on the

screen), redirecting data from one device to another (capturing

printer output in a disk file), and keeping programs from

interfering with each other during multitasking or program

switching. The program should also be able to dynamically modify a

particular device (something like TSRs in MS-DOS).

One of my top priorities in designing Kotekan was a consistent

program interface for all devices. For many tasks, such as

displaying a line of text, the program shouldn't have to care

whether it is writing to a video screen, printer, or disk file.

Error reporting should also be consistent. Figure 1 shows Kotekan's

standard virtual devices.

Figure 1, Virtual Devices

Operating System

System message video

System message keyboard

System error video

System error keyboard

Program message video

Program message keyboard

Program error video

Program error keyboard

Printer

Serial ports

Mouse

Disks

The Operating System device handles things not associated with any

particular device such as memory allocation, timing functions, and

loading and running programs. Although Kotekan supports separate

default keyboards and video displays for the operating system and

program, and separate error devices, these virtual devices

often share the same physical hardware. Kotekan's virtual device

support and object-oriented design make it very easy to redirect any

device to any other. A printer or serial port might be used for

error displays, or the virtual mouse device might hijack the

keyboard's direction keys.

During a system call, the program provides a device selection

parameter which ranges from 0 to one less than the maximum number of

devices supported. Actually each virtual device has two selection

parameters. An even value requests output, while an odd value

(output + 1) selects input. As far as the program is concerned,

input and output could be two separate and unrelated devices.

Kotekan uses the device selection parameter as an index into a

Master Dispatch Table which contains an output and input entry for

each defined device. Even though a keyboard is normally thought of

as an input device, the IBM AT type keyboard can accept commands for

such things as turning on LEDs and changing the key repeat rate.

When two virtual devices share the same physical device, case their

entries in the Master Dispatch Table will be identical.

An operating system should be able to handle service requests with

minimal processing overhead. Inheritance and device redirection

often require that devices share data and methods. The need for any

device to accept any service request means that undefined or invalid

requests must be trapped and handled appropriately. One resident

device might only provide 10 services, but a dynamically loaded

device might provide 20 services. What happens if a program

requests service 15 from the first device or dynamically modifies a

particular service request? Kotekan uses various techniques to

handle these situations.

Each entry in the Master Dispatch Table is a pointer to a Service

Parameter Block (see figure 2). Each entry in this block is another

pointer to a service routine or data block for the device. Similar

devices often share service routines or data blocks and thus may

have identical entries in their respective Service Parameter Blocks.

A program requests a particular service with a service parameter

which Kotekan uses as an index to the Service Parameter Block. As

part of the service routine, Kotekan loads a specific CPU register

with a pointer to the selected Service Parameter Block. Individual

service routines then use this register as a base pointer for

locating their local data and methods.

Figure 2, Service Parameter Block

Identification

Base Data Block

Physical Data Block

Virtual Data Block

Link Pointer

Method Data Block

Method Processor

String I/O

Character I/O

Software I/O

Software Status

Hardware I/O

Hardware Status

Error Handler

Interrupt Handler

32 character device name

The Identification field includes the device selection index, the

parent device selection index, and the total number of entries in

the Service Parameter Block. This count serves as an offset to the

device name, provides an upper limit for bounds checking and allows

a device to extend the Parameter Block as needed. The most common

service requests have their own entries in the Service Parameter Block.

These entries are pointers to the actual service routine. Kotekan

just uses the program's service parameter as an index into the

Service Parameter Block, reads the address and hops to it. Lookup

tables are fast, but are difficult to extend dynamically, and

require that every possible service request have an entry. They also

require that each device use the same indexes which can leave many

gaps for unsupported methods. For services beyond the basics,

Kotekan has a "method" service. The Method Processor entry holds the

address of the method service routine. This service requires a third

parameter which requests a specific virtual method. The Method

Processor searches a table pointed to by the Method Data Block entry

in the Service Parameter Block. This type of search is more time

consuming than an indexed lookup, but doesn't require any special

order within the method table and eliminates entries for unsupported

methods. The last entry in the Method Data Block contains the

address of a default service routine, which is called if no matching

method is found. Separate entries for the Method Processor and

Method Data Block allows extra flexibility when modifying or

redirecting devices.

The Service Parameter Block contains three different data blocks for

each device. The first word of each block is a size field which

allows each device to have a different block size. The Base Data

Block is common to all instances of a device and contains parameters

that don't change from one instance to the next, such as current

hardware control register settings. Several virtual devices all

sharing the same serial port or disk controller chip would also

share the same Base Data Block. The Physical Data Block contains

things that may change from one instance to another, but are related

in some way to external hardware. A screen may contain five

windows, each with its own default cursor position and colors, but

the physical display can only handle one at a time. As a new window

is selected, the cursor position and current display attributes

are updated from the selected Physical Data Block. The Virtual Data

Block contains parameters that are local to the virtual device and

not necessarily related to any physical device. This includes things

like tab expansion and page length. When a device is redirected, it

uses the Physical Data Block of the target device, but always keeps

its own Virtual Data Block.

The actual I/O routines consist of four levels, although not all

devices need all four. The hardware I/O and status routines are the

lowest level and directly access hardware. The software level is

optional and points directly to the hardware routines if not needed.

This level provides intermediate processing between the hardware and

character routines. Things like XMODEM serial protocol and buffering

are handled at this level. The character routine is basically a

translation service and therefore doesn't require a separate status

routine. The character level handles special control characters like

carriage returns, and updates the cursor position. The character,

software and hardware routines all use the same CPU register for

reading and writing characters. If one level isn't needed, its

Service Parameter Block entry can point directly to the next level.

The block level handles strings. The program can request either the

character routine or software routine for processing individual

characters. The block input service can optionally edit, echo to a

selected output device and convert lower case to upper case. The

string can be terminated by a specific character count, a designated

character, or during input, by a handshaking condition (such as an

end-of-file indication) from the physical device.

Programs can also request some services by sending special

characters (see figure 3) to the character output routine of a

device. This is a convenient way to clear the screen, define a

window or change video attributes, as these special characters can

be embedded in a string.

Figure 3, Special Characters

00-7f standard ASCII

80-9f virtual method control codes

a0-bf upper 5 bits of 13 bit character, next character supplies lower bits

c0-ff alternate 8 bit characters

Some Virtual Method Control Codes, 80-9f

80 = ignore this character

81 = take the next character literally

85 = the next byte is a sub-command

86 = set attributes according to next byte

Some Sub-commands for Virtual Method Control Code 85

23 = clear screen/new page

24 = character cursor position follows

25 = pixel cursor position follows

Putting It All Together

Now for a look at how all of this works. Figure 5 shows typical

Character and Block Output routines in pseudo-code. Figure 4 show the

pseudo-code conventions I use. This pseudo-code is not intended to

be a literal translation of the actual assembly language. It merely

illustrates some of the general principles involved at various

levels.

Figure 4, Pseudo-Code Conventions

RegPtr is a CPU registers which always points to the current Service

Parameter Block.

Items in parentheses are pointers. (RegPtr + Character Output) is the

address stored in the Character Output position of the Service

Parameter Block pointed to by RegPtr.

;;Semicolons begin a comment

IF condition

code to do if condition is true

ELSE

code to do if condition was false

ENDIF

UNTIL condition is true

code here is always done at least once

ENDU Check condition and branch back to UNTIL if false

CALL subroutine

RETURN from subroutine

Figure 5, Typical Character and Block Output routines

Video Display Character Output::

;; The character to display is assumed to be in a CPU register

IF character is 80-9f

;; Process virtual method code

Load CPU register with appropriate virtual method code

CALL (RegPtr + Method Processor)

ELSE

;; Just display character

UNTIL ready to send character

CALL (RegPtr + Software Status)

ENDU

CALL (RegPtr + Software Output)

IF character was carriage return

Set cursor position to start of line

ELSE

IF character was line feed

Drop down one line

ELSE

Advance cursor position

IF cursor past end of line

Wrap to next line

ENDIF

ENDIF

ENDIF

ENDIF

RETURN

Block Output::

;; The program passes a pointer and string length in CPU registers

UNTIL string length counter is 0

IF Character Output selected

CALL (RegPtr + Character Output)

ELSE

UNTIL ready to send character

CALL (RegPtr + Software Status)

ENDU

CALL (RegPtr + Software Output)

ENDIF

Advance pointer to next character

Decrement string length counter

ENDU

RETURN

Note that the pseudo-code in figure 5 will work for various devices.

Since each device has its own unique Service Parameter Block address

in RegPtr, the Character Output routine always sends the character to

the proper hardware device. Now suppose we want to send a character

to the printer. We could use the Video Display Character Output

routine of figure 5 and the character would get to the printer as

long as RegPtr pointed to the printer's Service Parameter Block, but

suppose we want to expand tabs and send a form feed every 60 lines?

We obviously need a separate Printer Character Output routine, but only

for tabs and line feeds. Figure 6 shows a pseudo-code routine that

would do the job.

Figure 6, Printer Character Output

Printer Character Output::

;; The character is assumed to be in a CPU register

IF character is a tab

Calculate number of spaces required

UNTIL all spaces displayed

Load CPU register with space character

CALL Video Display Character Output

ENDU

ELSE

CALL Video Display Character Output

IF character was a line feed

IF row counter = 60

Advance paper to next page

Reset row counter

ENDIF

ENDIF

ENDIF

RETURN

The Video Display Character Output handles almost everything. Since

row and column counters are kept in (RegPtr + Physical Data Block),

even the current position is properly maintained. Any new features

added to the Video Display Character Output routine are

automatically available to the Printer Character Output routine.

Even virtual methods are properly handled by the Video Device

Character Output routine. A printer can't clear its screen as a

video display does, but it can advance the paper to a new page.

Since service routine addresses are kept in (RegPtr + Method Data

Block), virtual methods are always handled by the selected device in

an appropriate way. While a video display might clear the screen and

select a new foreground color, a printer might advance to a new page

and switch to an italics font, and a disk drive would just store the

virtual method characters as is. When the disk file was later

displayed on the screen or printer, the virtual methods would be

properly interpreted. Polymorphism at work.

Inheritance among devices

So far, we have seen two ways in which one device can inherit from

another. Two devices with identical needs simply share the same

service routine. Devices such as printers can use another service

routine in most cases, providing a unique service routine only for

those needs that are actually different. Kotekan also supports

inheritance during initialization. Each device has an Initialization

Parameter Table, which specifies a parent device. Kotekan first

builds a default Service Parameter Block using the parent's data.

Additional initialization parameters specify which routines are to

be overridden by the descendent device. This procedure greatly

simplifies adding new device drivers as each device need only

provide that code and data which is different from the parent

device.

The ultimate ancestor of all devices is the null device. Anything

written to the null device falls into the bottomless bit bucket, and

any attempt to read from the null device returns an end-of-file

error. The null device's block and character routines can often be

inherited without modification, as can many methods.

Redirecting I/O and Dynamically Altering Devices

One of the null device's methods creates a new instance of a Service

Parameter Block on the heap and installs the address of this new

block in the Master Dispatch Table, making this new instance the

current default. Following initialization, the Base Data Block and

Physical Data Block entries usually point to the same address. When

Kotekan creates a new instance of the device, the Physical Data

Block points to the new instance's parameters while the Base Data

Block remains unchanged. All instances are kept in a linked list

along with additional information which allows Kotekan to remove an

instance and generally manage things efficiently. New instances can

be altered to temporarily expand or modify the device. When the

instance has served it's purpose, Kotekan simply changes the Master

Dispatch Table entry back to the parent instance and discards the

temporary instance. To redirect printer output to the video display,

Kotekan creates a new instance of the printer's Service Parameter

Block and changes most the entries to point to the video display's

data and methods. The Character I/O and Virtual Data Block are

retained as is, which allows the printer to retain essential

attributes such a tab expansion and a different line and page length

if desired. A program can select any specific instance by passing a

handle (the address of the desired Service Parameter Block) along

with any other required parameters.

Combining Devices

Sometimes a device needs some attributes of a different device short

of complete redirection. In the absence of a true mouse, Kotekan

can simulate a mouse using direction keys on the keyboard. As far

as the program is concerned, the mouse is there and all mouse

methods work as always, but the mouse device intercepts keyboard

input, snatches the direction keys and passes everything else

through as is. A kind of multiple inheritance makes all this

possible. Kotekan creates new instances of both the mouse and

keyboard devices. The mouse Hardware I/O and Hardware Status

routines call the keyboard Software I/O and Software Status

routines. The mouse then filters the incoming keypresses and

intercepts the direction keys. The keyboard routines call the mouse

Software I/O and Software Status routines and thus receive only

those keys passed through by the mouse device. The mouse device

Method Processor first searches the keyboard methods, and if nothing

is found, tries the mouse methods. Figure 7 illustrates how this

works.

Figure 7, Mouse Simulation

New Instance of Keyboard Service Parameter Block

Identification - Unchanged

Base Data Block - Unchanged

Physical Data Block - Unchanged

Virtual Data Block - Unchanged

Link Pointer - Unchanged

Method Data Block - If no match here, search Mouse Method Data Block too

Method Processor - Unchanged

String I/O - Unchanged

Character I/O - Unchanged

Software I/O - Unchanged

Software Status - Unchanged

Hardware I/O - Call new instance of Mouse Software I/O with Mouse RegPtr

Hardware Status - Call new instance of Mouse Software Status with Mouse RegPtr

Error Handler - Unchanged

Interrupt Handler - Unchanged

New Instance of Mouse Service Parameter Block

Identification - Unchanged

Base Data Block - Unchanged

Physical Data Block - Unchanged

Virtual Data Block - Unchanged

Link Pointer - Unchanged

Method Data Block - Unchanged

Method Processor - Unchanged

String I/O - Unchanged

Character I/O - Unchanged

Software I/O - New routine

Software Status - New routine

Hardware I/O - Call original Keyboard Software Input with Keyboard RegPtr

Hardware Status - Call original Keyboard Software Status with Keyboard RegPtr

Error Handler - Unchanged

Interrupt Handler - Unchanged

New Mouse Software Status routine

CALL (RegPtr + Hardware Status)

IF character is waiting

CALL (RegPtr + Hardware Input)

IF direction key

Update mouse position

ELSE

Store character in local buffer

ENDIF

ENDIF

See if anything in local buffer and set status

RETURN

New Mouse Software I/O routine

UNTIL character ready

CALL (RegPtr + Software Status)

ENDU

Read character from buffer

RETURN

Disk Drives

Disk files are not much different from other devices. Kotekan

simply creates a new instance for each open file. Any data transfer

defaults to the currently active file (the current entry in the

Master Dispatch Table) unless a new instance is selected or the

system call specifies that the handle passed by the program should

be used instead of the default. File instances descend from

directory instances, and subdirectory instances descend from the

root directory. A complete path name can be reconstructed simply by

following the linked list back to the root directory.

Of Mice and Menus

I have always had mixed feelings about graphical interfaces. I find

them very helpful when using a new program, or a program that I

don't use enough to remember all the commands, but once I learn an

application well, several layers of menus sometimes get in the way.

Some programs even obscure needed information with dialogue boxes.

Kotekan supports an experimental hybrid interface. Programs use a

traditional command line interface, but menus can "filter" keyboard

or mouse input to provide assistance when needed. Selecting a menu

item simply sends the corresponding keyboard command to the program,

just as if the command were typed directly. Another menu option can

pass parameters directly to a subroutine in the program. These

menus are objects in themselves and can be dynamically created,

modified, and cleared.

What Does It All Mean?

Objects provide a simple means of creating, modifying and managing

devices. Kotekan has no file control blocks, no maximum number of

file buffers - it simply expands until the heap is full. New device

drivers can be installed easily by inheriting most of their program

code from other devices. Kotekan's consistent device interface and

virtual methods allow programs to work with new or modified devices

without prior knowledge of specific hardware.

As with any substantial program Kotekan will never be done (just

like MS-DOS 5.0 and counting). So far, Kotekan's object code

occupies about 25K and will probably expand by at least 50% if I

have the time to complete all my desired features. Of course, each

new addition begets more ideas, expanding my "to do" list at the

bottom faster than I can shrink it at the top.

Biography

Don Rowe is a part-time mad scientist and full-time consultant

specializing in embedded controllers and real-time programming.

Digital Arts

P. O. Box 323

Julian CA, 92036

phone/fax 619-765-2463