Opcode summary by hex value

Check bits 0-1:

00 = byte

01 = word

11 = double word

Format 2-4

Check bits 2-3:

11 = format 2-3

Check bits 4-7:

111 = format 3

Format 3:  nnnnn nnnn 11111 ii

gen   op         integer type

000

011111ii - Call external procedure

001

011111ii - Bit clear in PSR

010

011111ii - Jump

011

011111ii - Bit set in PSR

101

011111ii - Adjust stack pointer

110

011111ii - Jump to subroutine

111

011111ii - Case branch

Format 2:  nnnnn nnnn nnn 11 nn

gen   short op    integer type

111 not allowed, looks like format 3

n00011ii - Add quick integer

n00111ii - Compare quick integer

n01011ii - Store processor register

n01111ii - Save condition as boolean

n10011ii - Add, compare and branch

n10111ii - Move quick integer

n11011ii - Load processor register

Format 4:  nnnnn n nnnn nnnn nn

gen1  ? gen2 op   integer type

nn11 not allowed, looks like format 2 or 3

nn0000ii - Add

nn0001ii - Compare

nn0010ii - Bit clear

nn0100ii - Add with carry

nn0101ii - Move

nn0110ii - OR

nn1000ii - Subtract

nn1001ii - Compute effective address

If ii=11 then Load external procedure descriptor

nn1010ii - AND

nn1100ii - Subtract with borrow

nn1101ii - Test bit

nn1110ii - Exclusive OR

10 = non-integer opcode:

Check bits 2-3:

00 = format 1

Format 1:  nnnn0010

op

00000010 - Branch to subroutine

00010010 - Return from subroutine

00100010 - Call external procedure

00110010 - Return from external procedure

01000010 - Return from trap

01010010 - Return from interrupt

01100010 - Save registers

01110010 - Restore registers

10000010 - Enter new context

10010010 - Exit context

10100010 - NOP

10110010 - Wait for interrupt

11000010 - Jump to subroutine

11010010 - Flag trap

11100010 - Supervisor call trap

11110010 - Breakpoint trap

10 = format 0

Format 0:  nnnn1010

cond

nnnn1010 - Conditional branch

11101010 - Unconditional branch

11 = formats 5-8

Check bits 4-5:

00 = formats 5-7

Check bits 6-7:

00 = format 5

Format 5:  00000 nnnn 0 nnnn nn 00001110

short  op   integer type

0000ii

00001110 - Move string

0001ii

00001110 - Compare string

0010ii

00001110 - Set configuration resister

0011ii

00001110 - Skip string

01 = format 6

Format 6:  nnnnn nnnnn nnnn ii 01001110

gen1  gen2  op   inteter type

0000ii

01001110 - Rotate

0001ii

01001110 - Arithmetic shift (L/R)

0010ii

01001110 - Invert bit

0011ii

01001110 - Another invert bit

0101ii

01001110 - Logical shift (L/R)

0110ii

01001110 - Set bit

0111ii

01001110 - Set bit too

1000ii

01001110 - Negate

1001ii

01001110 - NOT

1011ii

01001110 - Subtract packed decimal

1100ii

01001110 - Absolute value

1101ii

01001110 - Complement

1110ii

01001110 - Clear bit

1111ii

01001110 - Add packed decimal

11 = format 7

Format 7:  nnnnn nnnnn nnnn nn 11001110

gen1  gen2  op   integer type

0000ii

11001110 - Move multiple

0001ii

11001110 - Compare multiple

0010ii

11001110 - Insert field short

0011ii

11001110 - Extract field short

0100ii

11001110 - Move (sign extend byte to word)

0101ii

11001110 - Move (0 extend byte to word)

0110ii

11001110 - Move (0 extend byte to double word)

0111ii

11001110 - Move (sign extend to double word)

1000ii

11001110 - Multiply

1001ii

11001110 - Extended multiply

1011ii

11001110 - Extended divide

1100ii

11001110 - Quotient

1101ii

11001110 - Remainder

1110ii

11001110 - Divide mod

1111ii

11001110 - Divide

10 = format 8

Format 8:  nnnnn nnnnn nnn n nn nn 101110

gen1  gen2  reg   integer type

op   op

0nn

00101110 - Extract bit field

0nn

01101110 - Convert to bit pointer

0nn

10101110 - Insert field

0nn

11101110 - Check array index

1nn

00101110 - Calculate array index

1nn

01101110 - Find first set bit

############################