.pl 62

DEFINICON SYSTEMS, INC.

DSI-32 COPROCESSOR BOARD

& MS/PC DOS SYSTEM SOFTWARE

INSTANT GRATIFICATION

.pa

#Instant Gratification#

Now  that you have received your DSI-32 card and assembled it (if

it  was a kit) and you've plugged into your computer (please  see

the instructions on installation), you probably want some instant

gratification. Well, here it comes.

The  first  thing  you  should do is make a  backup  of  all  the

distribution  disks  that came with your board and then  put  the

originals in a safe place.  Next you should create a subdirectory

for  your  DSI-32 development area.  Into this  subdirectory  you

should copy all the files on your distribution disks.  Now we are

ready  to  try the DSI-32 out,  part a) of this  discussion  will

describe  how  to  try out the compilers and part b) is  for  the

people that bought the DSI-32 without any compilers.

a) Lets assume for now that you purchased the C compiler, on your

disk you will find a file called HELLO.C.  Type the file out  and

you will see a simple print statement with the hello message.  We

will now compile and run the test program.

A>CC HELLO                    ; run the compiler

A>AS HELLO                    ; assemble the compiler output

A>LN FILE=HELLO               ; link the assembler output

At this stage you will find several new files on your disk,  they

are:

HELLO.A32      which is the output from the compiler,  it is

in 32000 assembler code.

HELLO.O32      which is the output from the assembler, it is

the object file that is ready for linking.

HELLO.E32      which  is the executable file that the linker

produces,  this  is the file that is ready to

run.

Now we are ready to run our test program,  all that is  necessary

is to type:

A>LOAD HELLO

If everything went ok, the hello message will be displayed on the

console.  Now,  if  you want to play for a while just  experiment

with changing the message. Note if you have the Pascal or Fortran

compiler  instead  of  the C then all that changes  is  the  test

filename,  it would be HELLO.P or HELLO.F and the compile command

would be either:  A>PC HELLO or A>FC HELLO ,respectively. All the

rest of the commands to assemble, link and load are the same.

.pa

b) Now lets assume that all you purchased was the DSI-32, with it

comes  a  less elaborate (but object level compatible)  assembler

and linker.  So in your case we supply an assembler test  program

that says hello.  It is called HELLO.A32,  and uses direct system

calls to display the hello message.  Let's go through the process

of assembling and linking the test program.

A>AS HELLO                    ; assemble the test program

A>LN FILE=HELLO               ; link the test program

At this stage you will find several new files on your disk,  they

are:

HELLO.O32      which is the output from the assembler, it is

the object file that is ready for linking.

HELLO.E32      which  is the executable file that the linker

produces,  this  is the file that is ready to

run.

Now we are ready to run our test program,  all that is  necessary

is to type:

A>LOAD HELLO

If  everything went ok the hello message will be displayed on the

console.  Now,  if  you want to play for a while just  experiment

with changing the message.

At  this stage you should be suitably gratified and it  would  be

very wise for you to #read the rest of the manual# before you wreak

havoc and get disillusioned.