.pl 62

Chapter 3

DSI-32 PROGRAMS

#3.1  INTRODUCTION#

DSI-32  assembly  language programs and how the DSI-32  Assembler

assigns memory addresses to symbols,  instruction,  and data  are

covered in this Chapter.

#3.2  DSI-32 MODULE STRUCTURE#

A  DSI-32  program  may  be  divided  into  separately  assembled

modules.  A  module consists of one or more segments and has  the

following form:

[MODULE <symbol>]

<segment-1>

<segment-2>

.

.

.

<segment-n>

description:

MODULE <symbol>  is  the  optional  MODULE  pseudo-op.  If  it

appears  within  the  current  module   being

assembled  the string <symbol> is written  to

the object file as the module name, otherwise

the  name  of the source file is assigned  as

the module name.  Section 5.8.1 describes the

MODULE pseudo-op in detail.

<segment-1>      specify the segments within the module.  Each

thru           segment may consist of one or more  Assembler

<segment-n>      statements. Section 3.3 describes segments in

detail.

If a program is made up several modules,  the modules must all be

separately  assembled  and then linked together (via  the  DSI-32

linker).

.pa

#3.3  PROGRAM SEGMENTS#

A  program  segment is made up of a set of statements  which  all

have  the same relocation mode.  The following segment types  are

supported:

- Program Code

- Static Base

- Frame Pointer

- Stack Pointer

- Absolute

- Immediate

#3.3.1  Relocatable Addresses And Relocation Modes#

Each  byte  of  data storage defined in a module  is  assigned  a

relocatable address. A relocatable memory address is an offset to

the  start  of the segment in which the instruction or  data  was

defined.

Also associated with a relocatable memory address is a relocation

mode. The relocation mode specifies the base address to which the

address is relative.  The Assembler assigns a relocation mode  to

an  instruction  or  data element depending on the  type  of  the

segment in which it is defined or whether the relocation mode was

explicitly  defined.  Table  3-1 lists the  permitted  relocation

modes:

.cw 10

TABLE 3-1  RELOCATION MODES

-------------------------------------------------------------------------

|   Name  |	Mode type	|	Description			|

-------------------------------------------------------------------------

|    PC   |	Program code	| Value is relative to the start of the |

|	  |			| program code segment			|

|	  |			|					|

|    SB   |	Static Base	| Value is relative to the SB register  |

|	  |			| at run time.				|

|	  |			|					|

|    FP   |	Frame Pointer	| Value is relative to the FP register	|

|	  |			| at run time.				|

|	  |			|					|

|    SP   |	Stack Pointer	| Value is relative to the SP register	|

|	  |			| at run time.				|

|	  |			|					|

|    AB   |	Absolute	| Value is the absolute memory address.	|

|	  |			|					|

|    IM   |	Immediate	| Value is a constant.			|

-------------------------------------------------------------------------

.cw 12

.pa

#3.3.2  Program Code Segments#

Relocation Mode - PC Relative.

It is the only segment where code generation is allowed,  all the

constants  and  program code are generated in this  segment.  The

location  counter  is incremented by the  appropriate  number  of

bytes  after each statement.  The pseudo location counter may not

be decremented in the PC segment.  The program code segments  are

relocated  at  run  time via the Module table  mechanism  of  the

NS32000 series of microprocessors.

#3.3.3  Static Base Segments#

Relocation Mode - SB Relative.

Static  data  is referenced relative to the SB  segment.  The  SB

segment  is fixed as long as the program resides in  one  module.

The  SB  register  is loaded from the Module table  each  time  a

program enters a new module.

The  Assembler  pseudo  location counter is  incremented  by  the

appropriate  amount after each instruction.  The location counter

may not be decremented in the SB segment.

Only  uninitialized  storage may be allocated in the  SB  segment

when using the NSX compatible DSI-32 assembler. The extended DSI-

32  assembler  allows initialized storage which is output to  the

object file along with the code segments.

The static base segments are relocated at run time via the Module

table mechanism of the NS32000 series of microprocessors.

#3.3.4  Frame Pointer Relative#

Relocation Mode - FP Relative Segment.

- This  segment allocates storage for data items that are  local

to  a  procedure.   Storage of FP relative data is allocated  and

deallocated at run time.

- FP  relative data items are allocated on the stack,  thus  the

location  counter  is  decremented by the appropriate  number  of

bytes.  If a label is specified on a line which allocates storage

on  the stack,  it is assigned the value of the location  counter

after it has been decremented by the appropriate number of bytes.

- FP segments are on the stack and transient, thus Assembly time

initialization is not possible.

.pa

#3.3.5  Stack Pointer Relative Segment#

Relocation Mode - SP Relative

Data  that is relative to the stack is allocated by this segment.

SP relative data items are allocated on the push down stack, thus

the location counter is decremented by the appropriate number  of

bytes.  If a label is specified on a line which allocates storage

on  the  stack it is assigned the value of the  location  counter

after it has been decremented by the appropriate number of bytes.

SP  segments  like  FP and SB segments only contain  storage  for

data,  thus Assembly time initialization is not  possible.  Again

note  that  the extended DSI-32 assembler  allows  assembly  time

initialized SB segments.

An  SP relative segment is relocated at run time by  instructions

which modify the stack pointer

#3.3.6  Absolute Segment#

Relocation Mode - Absolute.

This segment allocates storage at absolute memory locations.  The

absolute  location  counter  is incremented  by  the  appropriate

number of bytes.

This  segment  is  not relocatable since it  refers  to  absolute

addresses.   Input/output ports can be easily reference via  this

addressing mode.

#3.3.7  Immediate Segments#

Relocation Mode - Immediate.

Storage  is not allocated by this segment.  The immediate segment

is a mechanism for assigning immediate values to labels which may

then  be  used as offsets in the various  addressing  modes.  The

Immediate  location  counter is incremented  by  the  appropriate

number of bytes.

This  segment is not relocatable since it doesn't refer to memory

locations.

#3.4  NESTED SEGMENTS#

The  Assembler supports the nesting of segments,  i.e.  it allows

defining a new segment within the current segment.   The  maximum

nesting level is 16.   Note that the PC and SB segments even when

nested  continue to increment from the last used value unlike the

FP and SP segments which are reset on every new occurrence.